K1-SB - Large format low-end extension

K Series

Low-end extension

Deployment in vertical lines

Omni configuration

# K1-SB

## Short description

Large format low-end extension featuring two 15 in speakers with high excursion drivers and optimized laminar vents to maximize LF efficiency - deployed on its own as a subwoofer or in-line with large format loudspeakers to extend low-frequencies throw capacity

## Description

Passive large format low-end extension with two high-excursion, direct-radiating 15 in transducers mounted in a bass-reflex tuned enclosure with laminar vents for increased LF efficiency. The enclosure cabinet is constructed of premium grade Baltic beech and birch plywood. The enclosure front is protected by a coated steel grill and an acoustically transparent fabric. The enclosure integrates four ergonomic handles and a four-point flush-fitting hardware for rigging and combining with other loudspeakers of the same family. Exclusively driven and protected by a proprietary amplified controller through a dedicated preset.

## Technical requirements

Coverage:

- Enclosure directivity: omnidirectional

Transducers:

- LF transducer: 2 × 15" cone drivers

Acoustics:

- Nominal impedance: 4 Ω

- Usable bandwidth: 30 Hz - 80 Hz (-10 dB), 35 Hz - 70 Hz (-6 dB), 38 Hz - 61 Hz (-3 dB)

- RMS power handling (calculated using the mean impedance measured on the usable bandwidth): 930 W

- Maximum SPL: 141 dB (peak level at 1 m under half space conditions using pink noise with crest factor 4)

Physical data:

- Acoustical load: bass-reflex

- Cabinet: premium grade Baltic birch plywood

- Dimensions (W, H, D): 1342 mm, 438 mm, 520 mm / 52.8 in, 17.1 in, 20.5 in

- Weight (net): 83 kg / 183 lb

- Connectors: IN: 1 × 4-point speakON

- IP: IP45

- Finish: dark grey brown Pantone 426 C

Product name: K1-SB

Manufacturer: L-Acoustics