# K1 - Long throw line source

K Series

Long throw > 35 m

Deployment in vertical lines

Incremental coverage with variable inter-element angles

# K1

## Short description

3-way active enclosure with two 15 in drivers, four 6.5 in and three 3 in diaphragm compression driver - 5-degree enclosure angle with a 90-degree waveguide directivity for assembling enclosures to achieve a coherent wavefront line source.

## Description

3-way active enclosure with two weather-resistant premium 15 in transducers, four 6.5 in medium drivers and three high-output 3 in diaphragm compression driver coupled to a cylindrical waveguide with 90 degrees of horizontal directivity and 5 degrees of vertical coverage. The enclosure cabinet is constructed of premium grade Baltic beech and birch plywood. The enclosure front is protected by a coated steel grill and an acoustically transparent fabric. The enclosure integrates four ergonomic handles. A four-point flush-fitting hardware for rigging and combining with other loudspeakers of the same family to achieve a coherent wavefront line source. The line source directivity depends upon the number of elements and inter-element angles and can be matched to the audience geometry. Exclusively driven and protected by a proprietary amplified controller through a dedicated preset.

## Technical requirements

Coverage (-6 dB):

- Enclosure directivity: dependent upon the number of elements and the line source curvature

- Waveguide directivity: 90° symmetric

Transducers:

- LF transducer: 2 × 15" cone drivers

- MF transducer: 4 × 6.5" cone drivers

- HF transducer: 3 × 3" diaphragm compression driver

Acoustics:

- Nominal impedance:

  - LF: 2 × 8 Ω

  - MF:  8 Ω

  - HF:  8 Ω

- Usable bandwidth: 35 Hz - 20 kHz (-10 dB), 38 Hz - 19 kHz (-6 dB), 42 Hz - 19 kHz (-3 dB)

- RMS power handling (Calculated using the mean impedance measured on the usable bandwidth):

  - LF:  422 W

  - MF:  497 W

  - HF:  199 W

- Maximum SPL: 149 dB (Peak level measured at 1 m under free field conditions using pink noise with crest factor 4)

Physical data:

- Acoustical load: bass-reflex

- Cabinet: premium grade Baltic birch plywood

- Dimensions (W, H, D): 1342 mm, 438 mm, 520 mm / 52.8 in, 17.2 in, 20.5 in

- Weight (net): 106 kg / 234 lb

- Connectors:

  - IN:  1 × 8-point PA-COM

  - LINK:  1 × 8-point PA-COM

- IP: IP43

- Finish:

  - grey brown RAL 8019

Product name: K1

Manufacturer: L-Acoustics