K2 - Long throw line source

K Series

Long throw > 35 m

Deployment in vertical lines

Incremental coverage with variable inter-element angles

# K2

## Short description

3-way active enclosure with two 12 in drivers, four 6.5 in and two 3 in diaphragm compression driver - 10-degree enclosure angle with adjustable waveguide directivity for assembling enclosures to achieve a coherent wavefront line source.

## Description

3-way active enclosure with two weather-resistant premium 12 in transducers, four 6.5 in medium drivers and two high-output 3 in diaphragm compression driver coupled to a cylindrical waveguide with user-adjustable horizontal directivity and 10 degrees of vertical coverage. The enclosure cabinet is constructed of premium grade Baltic beech and birch plywood. The enclosure front is protected by a coated steel grill and an acoustically transparent fabric. The enclosure integrates two ergonomic handles. A four-point flush-fitting hardware allows for rigging and combining with other loudspeakers of the same family to achieve a coherent wavefront line source. The line source directivity depends upon the number of elements and inter-element angles and can be matched to the audience geometry. Exclusively driven and protected by a proprietary amplified controller through a dedicated preset.

## Technical requirements

Coverage:

- Enclosure directivity: dependent upon the number of elements and the line source curvature

- Waveguide directivity: 110°/70° symmetric or 90° asymmetric (35°/55° or 55°/35°)

Transducers:

- LF transducer: 2 × 12" cone drivers

- MF transducer:4 × 6.5"cone drivers

- HF transducer: 2 × 3" diaphragm compression driver

Acoustics:

- Nominal impedance:

  - LF:  2 × 8 Ω

  - MF:  8 Ω

  - HF:  16 Ω

- Usable bandwidth: 35 Hz - 20 kHz (-10 dB), 38 Hz - 20 kHz (-6 dB), 40 Hz - 20 kHz (-3 dB)

- RMS power handling (Calculated using the mean impedance measured on the usable bandwidth):

  - LF:  311 W

  - MF:  324 W

  - HF:  117 W

- Maximum SPL: 147 dB (Peak level measured at 1 m under free field conditions using pink noise with crest factor 4)

Physical data:

- Acoustical load: bass-reflex

- Cabinet: premium grade Baltic birch plywood

- Dimensions (W, H, D): 1338 mm, 354 mm, 400 mm / 52.7 in, 13.9 in, 15.8 in

- Weight (net): 56 kg / 123.2 lb

- Connectors:

  - IN:  1 × 8-point PA-COM

  - LINK:  1 × 8-point PA-COM

- IP: IP55

- Finish:

  - dark grey brown Pantone 426 C

Product name: K2

Manufacturer: L-Acoustics