Syva Low - Subwoofer

S Series

2 x 12' Low-end extension

Stacked deployment

Omni configuration

# Syva Low

## Short description

Passive low-end extension featuring two 12 in speakers with high-excursion drivers and optimized laminar vents to maximize LF efficiency - deployed in omnidirectional patterns in stacked configurations

## Description

Passive low-end extension with two high-excursion, direct-radiating 12 in transducers mounted in a bass-reflex tuned enclosure with laminar vents for increased LF efficiency. The enclosure cabinet is constructed of premium grade Baltic beech and birch plywood. The enclosure front is protected by a coated steel grill and an acoustically transparent fabric. The enclosure integrates a signal connector on the top panel to connect to the companion full-range enclosure. Exclusively driven and protected by a proprietary amplified controller through a dedicated preset.

## Technical requirements

Coverage:

- Enclosure directivity: standard

Transducers:

- LF transducer: 2 ×12" cone drivers

Acoustics:

- Nominal impedance: 4 Ω

- Usable bandwidth: 40 Hz - 130 Hz (-10 dB), 42 Hz - 110 Hz (-6 dB), 47 Hz - 93 Hz (-3 dB)

- RMS power handling (Calculated using the mean impedance measured on the usable bandwidth): 894 W

- Maximum SPL: 137 dB Peak level at 1 m under half space conditions using pink noise with crest factor 4

Physical data:

- Acoustical load: bass-reflex

- Cabinet: premium grade Baltic beech and birch plywood

- Dimensions (W, H, D): 849 mm, 334 mm, 350 mm / 33.4 in, 13.1 in, 13.8 in

- Weight (net): 29 kg / 64 lb

- Connectors:

  - 1 × 4-point speakON

  - 1 x proprietary connector

- IP: IP55

- Finish:

  - dark grey brown Pantone 426 C

  - pure white RAL 9010

  - custom RAL code on special order

Product name: Syva Low

Manufacturer: L-Acoustics