Kara IIi - Long throw line source

K Series Install

Long throw > 35 m

Deployment in vertical lines

Incremental coverage with variable inter-element angles

# Kara IIi

## Short description

Install-specific 2-way active enclosure with two 8 in drivers and one 3 in diaphragm compression driver - 10-degree enclosure angle with adjustable waveguide directivity for assembling enclosures to achieve a coherent wavefront line source.

## Description

Install-specific 2-way active enclosure with two weather-resistant premium 8 in transducers and high-output 3 in diaphragm compression driver coupled to a cylindrical waveguide with user-adjustable horizontal directivity and 10° of vertical coverage. The enclosure cabinet is constructed of premium grade Baltic beech and birch plywood. The enclosure front is protected by a coated steel grill and an acoustically transparent fabric. The enclosure integrates inserts for flush-fitting connecting plates and an optional acoustically transparent front screen. The four-point hardware allows for rigging and combining with other loudspeakers of the same family to achieve a coherent wavefront line source. The line source directivity depends upon the number of elements and inter-element angles and can be matched to the audience geometry. Exclusively driven and protected by a proprietary amplified controller through a dedicated preset.

## Technical requirements

Coverage (-6 dB):

- Enclosure directivity: dependent upon the number of elements and the line source curvature

- Waveguide directivity: 70° / 110° symmetric or 90° asymmetric (35°/90°)

Transducers:

- LF transducer: 2 x 8" cone drivers

- HF transducer: 3" diaphragm compression driver

Acoustics:

- Nominal impedance: 8 Ω LF, 8 Ω HF

- Usable bandwidth: 55 Hz - 20 kHz (-10 dB), 63 Hz - 20 kHz (-6 dB), 80 Hz - 19 kHz (-3 dB)

- RMS power handling (Calculated using the mean impedance measured on the usable bandwidth):

  - LF:  280 W

  - HF:  75 W

- Maximum SPL: 142 dB (Peak level measured at 1 m under free field conditions using pink noise with crest factor 4)

Physical data:

- Acoustical load: bass-reflex

- Cabinet: premium grade Baltic birch plywood

- Dimensions (W, H, D): 701 mm, 252 mm, 409 mm / 27.6 in, 9.9 in, 16.1 in

- Weight (net): 21 kg / 46 lb

- Connectors: 2 × 4-point terminal block with push-in connection

- IP: IP55

- Finish:

  - dark grey brown Pantone 426 C

  - pure white RAL 9010

  - custom RAL code on special order

Product name: Kara IIi

Manufacturer: L-Acoustics