Syva Sub - Subwoofer

S Series

12' Subwoofer

Stacked deployment

Omni configuration

# Syva Sub

## Short description

Passive subwoofer featuring one 12 in speakers with high-excursion driver and optimized laminar vents to maximize LF efficiency - deployed in omnidirectional patterns in stacked configurations

## Description

Passive subwoofer with a high-excursion, direct-radiating 12 in transducer mounted in a bass-reflex tuned enclosure with laminar vents for increased LF efficiency. The enclosure cabinet is constructed of premium grade Baltic beech and birch plywood. The enclosure front is protected by a coated steel grill and an acoustically transparent fabric. The enclosure integrates a signal connector on the top panel to connect to the companion full-range enclosure. Exclusively driven and protected by a proprietary amplified controller through a dedicated preset.

## Technical requirements

Coverage:

- Enclosure directivity: standard

Transducers:

- LF transducer: 1 × 12" cone driver

Acoustics:

- Nominal impedance: 8 Ω

- Usable bandwidth: 27 Hz - 120 Hz (-10 dB), 29 Hz - 100 Hz (-6 dB), 33 Hz - 80 Hz (-3 dB) / 29 Hz - 200 Hz (-10 dB), 32 Hz - 150 Hz (-6 dB), 37 Hz - 105 Hz (-3 dB)

- RMS power handling (Calculated using the mean impedance measured on the usable bandwidth): 250 W

- Maximum SPL: 128 dB Peak level at 1 m under half space conditions using pink noise with crest factor 4

Physical data:

- Acoustical load: bass-reflex

- Cabinet: premium grade Baltic beech and birch plywood

- Dimensions (W, H, D): 849 mm, 334 mm, 350 mm / 33.4 in, 13.1 in, 13.8 in

- Weight (net): 27 kg / 60 lb

- Connectors:

  - 1 × 4-point speakON

  - 1 x proprietary connector

- IP: IP55

- Finish:

  - dark grey brown Pantone 426 C

  - pure white RAL 9010

  - custom RAL code on special order

Product name: Syva Sub

Manufacturer: L-Acoustics