Syva - Medium throw colinear source

S Series

Medium throw: 35 m

Single element deployment

Line source directivity - Narrow vertically, wide horizontally

# Syva

## Short description

2-way passive enclosure with six 5 in drivers and three 1.75 in diaphragm compression drivers coupled to a waveguide to achieve a coherent wavefront with a 140° horizontal by 26° vertical coverage.

## Description

2-way passive enclosure with six 5 in drivers and three 1.75 in diaphragm compression drivers coupled to a cylindrical waveguide with 140° of horizontal directivity and a tightly controlled 26° of vertical directivity. The enclosure cabinet is constructed of premium grade Baltic beech and birch plywood with a slim elegance for easy integration. The enclosure front is protected by a coated steel grill and an acoustically transparent fabric. The enclosure integrates rigging and safety inserts for flown, wall-mount, pole-mount and stacked configuration. The enclosure integrates a signal connector on the bottom panel to connect to the companion subwoofer. Exclusively driven and protected by a proprietary amplified controller through a dedicated preset.

## Technical requirements

Coverage (-6 dB):

- Vertical directivity: +5°/-21°

- Horizontal directivity: 140°

Transducers:

- MF transducer: 6 x 5" cone drivers

- HF transducer: 3 x 1.75" diaphragm compression drivers

Acoustics:

- Nominal impedance: 8 Ω

- Usable bandwidth: 87 Hz - 20 kHz (-10 dB), 92 Hz - 20 kHz (-6 dB), 102 Hz - 20 kHz (-3 dB)

- RMS power handling (Calculated using the mean impedance measured on the usable bandwidth): 454 W

- Maximum SPL: 137 dB (Peak level measured at 1 m under free field conditions using pink noise with crest factor 4)

Physical data:

- Acoustical load: bass-reflex

- Cabinet: premium grade Baltic beech and birch plywood

- Dimensions (W, H, D): 144 mm, 1304 mm, 209 mm / 5.7 in, 51.3 in, 8.2 in

- Weight (net): 21 kg / 46 lb

- Connectors:

  - 1 × 4-point speakON

  - 1 × 2-point screw terminal

  - 1 x proprietary connector

- IP: IP54

- Finish:

  - dark grey brown Pantone 426 C

  - pure white RAL 9010

  - custom RAL code on special order

Product name: Syva

Manufacturer: L-Acoustics